ROUTINE TEST RECORD FOR WEIGH FEEDER/SCREW FEEDER

JOB No.	:						CUSTOMER:

CAPACITY	:				TPH		MATERIAL :

| WEIGH FEEDER | CONTROLLER | LOADCELL | TACHO / PROXIMITY | JUNCTION BOX |
| --- | --- | --- | --- | --- |
| Model No. Serial No. LOCAL PANEL/PUSH BUTTON STATION: Model No. Serial No. | Model No. Serial No. SW version | Model No. Serial No. 1) 2) 3) 4) | AC / Pulse Model No. Serial No. | Model No. Serial No. Model No. Serial No. Model No. Serial No. |

| AC/DC DRIVE Details | Motor | Gearbox | RAL details |
| --- | --- | --- | --- |
| Make: Model: Serial No.: Capacity: kW | Make: Model No.: Serial No. : Rating: kW | Make: Ratio: Model No.: Serial No. : | Make: Model No. : Serial No. : |
| NOTE: Check for short between Line & Neutral, Neutral & Earth, Line & Earth before powering ON the system |  |  |  |

| SL. No. | Test Performed | Specified Values | Actual Values |
| --- | --- | --- | --- |
| 1. | Input Supply Voltage (+/-10%) |  |  |
| 2. | SMPS Output (+/-5%) |  |  |
| 3. | Check for the Display, Touchpad/Keypad functionality |  |  |
| 4. | Belt/Screw Load as specified in G.A. or MPL |  | kg/m |
| 5. | Belt Speed as specified in G.A. or MPL |  | m/s |

Resolution: Load:           kg/m              Speed:                m/s        Rate:               tph           totalizer:                     tonnes

- Measurement to be done from 10 to 100% of the Feeder capacity unless specified in Design Document.

| Belt Length /width | mm. |
| --- | --- |
| Actual speed (Measured with stop clock) | m/s |

II). Speed calibration: Calibrate the speed by entering the actual belt speed when the Drive output is 100%.

| Drive output in % | Calculated Speed m/s | Measured Speed m/s |
| --- | --- | --- |
| 10 |  |  |
| 25 |  |  |
| 50 |  |  |
| 75 |  |  |
| 100 |  |  |

ROUTINE TEST RECORD FOR WEIGH FEEDER/SCREW FEEDER

III). Load Calibration:   Weight of Calibration Chain:			Kg/m

IV). VOLUMETRIC MODE:

V). GRAVIMETRIC MODE: Operating Range:

| Set rate tph |  | Actual rate tph Min Max Avg | Load in Kg/m | Totalizer (6 min) |  | % Error | Current O/p mA |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Act. | Meas. |  |  |
| 10% |  |  |  |  |  |  |  |
| 25% |  |  |  |  |  |  |  |
| 50% |  |  |  |  |  |  |  |
| 75% |  |  |  |  |  |  |  |
| 100% |  |  |  |  |  |  |  |

| Functionality Check - Control Panel Mode - Local Mode - Remote Mode |  |
| --- | --- |
| Check Digital Input functions |  |
| Check the function for PF Contacts from/to Customer |  |
| Check for the function Analog Input and outputs |  |
| Check Communication Specify the Slave Address, Baud Rate and other Communication Parameters |  |

Remarks:

Instrument Used:

Tested by:	 								Approved by:

Date:									(HOD – PDN)